SCB-4000 HIGH RESOLUTION DAY/NIGHT CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

A.	All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

B.	All systems and components shall have been thoroughly tested and proven in actual use.

C.	All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer.  The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

D.	All systems and components shall be provided with a one-day turnaround repair express and 24-hour parts replacement. The repair and parts express shall be guaranteed by the manufacturer on warranty and nonwarranty items.

2.02	SCB-4000 HIGH RESOLUTION DAY/NIGHT CAMERA

A.	The camera shall provide a horizontal resolution of 600 TV lines of high resolution color video and a minimum illumination in low light performance to 0.1 Lux. The camera shall be equipped with the a1 chipset and shall use extended dynamic range (XDR) technology.

B. 	The camera shall support intelligent video recording based on fixed or moved objects, fence line and fence area detection, and object tracking. The camera shall support dynamic noise reduction (DNR) for improved video with motion detection.

C. 	The camera shall support coaxial communication for access to an on-screen programmable display.

D.	The OSD shall be available in English, French, German, Spanish, Italian, Portuguese, Chinese, Taiwanese, Japanese, Korean, Russian, Polish, Czech, Romanian, Serbian, Swedish, Danish, and Turkish.

2.03	ELECTRICAL SPECIFICATIONS

A. Input Voltage	Dual 12V DC, 24V AC

B. Power Consumption	Max. 4.0 W

2.04	VIDEO SPECIFICATIONS

A. Imaging Device	1/2” Ex-view, HAD CCD

B. Total Pixels (Horizontal x Vertical)

1. SCB-4000 NTSC	811 x 508

2. SCB-4000 PAL	795 x 582

C. Effective Pixels (Horizontal x Vertical)

1. SCB-4000 NTSC	768 x 494

2. SCB-4000 PAL	752 x 582

D. Scanning System	2:1 Interlace

E. Synchronization	Internal, Line lock

F. Frequency (Horizontal, Vertical)

1. SCB-4000 NTSC	15.734KHz, 59.94Hz

2. SCB-4000 PAL	15.625KHz, 50Hz

G. Horizontal Resolution	Color: 600 TV Lines (Min.)
		B/W: 700 TV Lines (Min.)

J. Min. Illumination	Color: 0.1 Lux (50IRE@F1.2),
0.0002 Lux (Sens-up 512x)
B/W: 0.01 Lux (50 IRE@F1.2), 0.0002 Lux

K. S / N (Y Signal)	52dB  (Weight On, AGC Off)

L. Video Output	CVBS : 1.0 Vp-p, 75Ω composite

2.05	OPERATIONAL SPECIFICATIONS

A. On-Screen Display

1. NTSC	English, Korean, Japanese, Spanish, French, 
		Portuguese, Taiwanese

2. PAL	English, Chinese, German, Italian, French, Spanish, 
		Russian, Polish, Czech, Danish, Turkish

B. Camera Title	Off, On (Display 54 ea, 2 lines)

C. Day & Night	Auto (ICR), Extern, Color, B/W

D. Digital Noise Reduction	On, Off

E. Backlight Compensation	BLC (On, Off)

F. eXtended Dynamic Range	Low, Medium, High, Off

G. Digital Image Stabilizing	On, Off

H. Privacy Masking	On, Off (12 programmable zones, polygonal type)

I. 2D+3D Noise Filter (A1)	Off, Very low, Low, Medium, High, Very high, User, Fix

J. Sens-up (Frame Integration)	On, Off (Selectable limit ~ 512x)

K. Gain Control	On, Off

L. White Balance	ATW1, ATW2, AWC (Manual), 3200K, 5600K

M. Electronic Shutter Speed

1. NTSC	1/60 ~ 1/10,000 sec

2. PAL	1/50 ~ 1/10,000 sec

N. Digital Zoom	Off, On (2x ~ 16x)

O. Intelligent Video	On, Off (Fixed/Moved object, Fence Line or Area, 
		Tracking)

P. PIP	On, Off

Q. Communication	RS-485, Coaxial control (CCVC)

R. Protocol	Samsung-T, Samsung-E, Pelco-D, Pelco-P, Bosch, 
		Honeywell, Vicon, Panasonic

2.06	ENVIRONMENTAL SPECIFICATIONS

A. Operating Temperature	-10°C ~ +50°C (+14°F ~ +122°F)

B. Operating Humidity	Less than 90% RH

2.07	MECHANICAL SPECIFICATIONS

A. Dimension	64 x 58 x 109.2 (2.52” x 2.28” x 4.3”)

B. Weight	305 g (0.67 lb)

C. Lens Mount	C / CS

2.08 	CERTIFICATIONS

A. NTSC	FCC (Class A), UL (External power supply)

B. PAL	CE (Class A)

2.09	WARRANTY

A. Three years, parts and labor