SCB-3000/3001 COLOR CCD DAY/NIGHT CAMERA

TECHNICAL SPECIFICATIONS	SECURITY SYSTEM

DIVISION – 28 ELECTRONIC SAFETY AND SECURITY

LEVEL 1__28 20 00 ELECTRONIC SURVEILLANCE

LEVEL 2__28 23 00 VIDEO SURVEILLANCE

LEVEL 3__28 23 29 VIDEO SURVEILLANCE REMOTE DEVICES AND SENSORS

PART 2 – PRODUCTS

2.01	GENERAL

- All equipment and materials used shall be standard components that are regularly manufactured and used in the manufacturer’s system.

- All systems and components shall have been thoroughly tested and proven in actual use.

- All systems and components shall be provided with the availability of a toll-free (U.S. and Canada), 24-hour technical assistance program (TAP) from the manufacturer.  The TAP shall allow for immediate technical assistance for either the dealer/installer or the end user at no charge for as long as the product is installed.

- All systems and components shall be provided with a one-day turnaround repair express and 24-hour parts replacement. The repair and parts express shall be guaranteed by the manufacturer on warranty and nonwarranty items.

2.02	SCB-3000/3001 COLOR CCD DAY/NIGHT CAMERA

- The camera shall provide 600 TV lines of high resolution video and a minimum illumination in low light performance to 0.1 Lux (F1.2, color). The camera shall provide 2D and 3D filtering noise reduction control based on the super-sensitive noise reduction technology.

- The camera shall provide ICR day and night functions and shall provide 128x wide dynamic range focus. The camera shall support picture in picture technology (PIP).

- The camera shall support intelligent video recording based on motion detection, object tracking, fixed/moved objects, and alarm outage. The camera shall support privacy masking with a minimum of 12 programmable zones.

- The camera shall support coaxial communication for access to an on-screen programmable display. The OSD shall be available in English, French, German, Spanish, Italian, Portuguese, Chinese, Taiwanese, Japanese, Korean, Russian, Polish, Czech, Romanian, Serbian, Swedish, Danish, and Turkish.

2.03	ELECTRICAL SPECIFICATIONS

- A. Input Voltage	Dual 12V DC, 24V AC ± 10%

- B. Power Consumption	Max. 4.0 W

2.04	VIDEO SPECIFICATIONS

- A. Imaging Device

- 1. SCB-3001 NTSC	1/3”, 520K, CCD

- 2. SCB-3001 PAL	1/3”, 612K CCD

- 3. SCB-3000 NTSC	1/3”, 410K, CCD

- 4. SCB-3000 PAL	1/3”, 470K CCD

- B. Total Pixels

- 1. SCB-3001 NTSC	1028(H) x 508(V)

- 2. SCB-3001 PAL	1028(H) x 596(V)

- 3. SCB-3000 NTSC	811(H) x 508(V)

- 4. SCB-3000 PAL	795(H) x 508(V)

- C. Effective Pixels

- 1. SCB-3001 NTSC	976(H) x 494(V)

- 2. SCB-3001 PAL	768(H) x 494(V)

- 3. SCB-3000 NTSC	976(H) x 582(V)

- 4. SCB-3000 PAL	752(H) x 494(V)

- D. Scanning System	2:1 Interlace, Progressive

- E. Synchronization	Internal, Line lock, Extern

- F. Frequency

- 1. SCB-3001 NTSC	H: 15.734KHz, V: 59.94Hz

- 2. SCB-3001 PAL	H: 15.625KHz, V: 50Hz

- 3. SCB-3000 NTSC	H: 15.734KHz, V: 59.94Hz

- 4. SCB-3000 PAL	H: 15.625KHz, V: 50Hz

- G. Horizontal Resolution

- 1. SCB-3001	Color: 650 TVL, B/W: 700 TVL

- 2. SCB-3000	Color: 600 TVL, B/W: 700 TVL

- H. Min. Illumination

- 1. SCB-3001	Color: 0.1 Lux (50 IRE@F1.2),

- 0.0002 Lux (Sens-up 2. 512x)

- B/W: 0.01 Lux (50 IRE@F1.2),

- 0.0002 Lux (Sens-up 512x)

- 2. SCB-3000	Color: 0.03Lux (50 IRE@F1.2),

- 0.0006 Lux (Sens-up 512x)
B/W: 0.01 Lux (50 IRE@F1.2),

- 0.0002 Lux (Sens-up 512x)

- I. S / N (Y signal)	52dB  (Weight On, AGC OFF)

- J. Video Output	CVBS: 1.0 Vp-p, 75Ω composite

2.05	OPERATIONAL SPECIFICATIONS

- A. Iris Adjustment	DC, Video

- B. Electronic Shutter Speed

- 1. SCB-3001 NTSC	1/60 ~ 1/120k sec

- 2. SCB-3001 PAL	1/50 ~ 1/120k sec

- 3. SCB-3000 NTSC	1/60 ~ 1/120k sec

- 4. SCB-3000 PAL	1/50 ~ 1/120k sec

- C. Shutter Mode	Esc, Manual, A.FLK, Extern, Fixed

- D. On-Screen Display

- 1. NTSC	English, Korean, Japanese, Spanish, French,

- Portuguese, Taiwanese

- 2. PAL	English, Chinese, German, Italian, French, Spanish,

- Russian, Polish, Czech, Romanian, Serbian, Swedish,

- Danish, Turkish, Portuguese

- E. Gain Control	Low, High, Off

- F. Sens-Up (Frame Integration) 	Off, Auto (Selectable 2x ~ 512x)

- G. White Balance	ATW, Outdoor, Indoor, Manual, AWC (1,700k~11,000 K)

- H. Backlight Compensation	Off, User BLC, HLCc, SSDR, WDR

- I. 3D Noise Filter (SSNRIII)	Off, On (Level adjustable)

- J. Day & Night	AUTO, EXTERN, COLOR, B/W

- K. Profile	BASIC, DA/NIGHT, BACKLIGHT, ITS, INDOOR, USER

- L. Digital Zoom	Off, On  (x1 ~ x16)

- M. Sharpness	Off, On  (Level adjustable)

- N. Camera Title	Off, On  (Display 15 Characters

- O. SYNC	Int, Line-Lock, Extern

- P. VPS (Virtual Progressive Scan)	Off, On

- Q. Intelligence	Detection, Tracking, Fixed/Moved, Alarm Out

- R. ETC	PIP, Reverse

- S. Privacy Masking	Off, On (12 programmable zones/ Polygonal type)

- T. Digital Image Stabilizing (DIS)	Off, On

- U. Coaxial Communication	Pelco Coaxitron

- V. RS-485 Communication	Samsung-T, Samsung-E, Pelco-D, Pelco P, Bosch,

- Honeywell, Vicon, Panasonic

2.06	ENVIRONMENTAL SPECIFICATIONS

- A. Operating Temperature	-10°C ~ +50°C (+14°F ~ +122°F)

- B. Operating Humidity	Less than 90% RH

2.07	MECHANICAL SPECIFICATIONS

- A. Lens Mount	C, CS Mount

- B. Focus Adjustment	Manual

- C. Dimension	66(W) x 61(H) x 102(D)mm

- D. Weight	310 g

2.08	WARRANTY

- Three years, parts and labor